Final #3 Solution

- Determining drainage area

  - Fill Pits

  - Flow Direction

  - Flow Accumulation (fac)

  - Load the Outlets Shapefile and identify the flow accumulation value at the outlet

620523 grid cells x 30 x 30 m = 558.4 x 106 m2.  This is the drainage area.

- Delineating stream network

  - Watershed upstream of outlet

  - Reclassify to have a value of 1

  - Raster calculator (fac>1000)*wshed

  - Stream link

  - Stream to Feature making sure output is a feature class in a geodatabase so that the shapes have a shape length attribute

  - Evaluate statistics on the Shape Length

The total length of streams is 404064 m = 404 km.

- Preparing layout

  - Add snotel.xls

  - Display XY data using NAD 1983.prj

  - Export to Shapefile

  - Label

  - See layout next page

- Finding snotel elevations

  - Zonal Statistics as Table

  - Join to Shapefile attribute table

  - Export table to DBF and use Excel to prepare for presentation

- The following figure was generated using Excel

- Use Raster calculator to apply the formula 0.0107 elevation -18.12 to estimate SWE

("logan.tif" * 0.0107 - 18.12) * "wshed1".  Isolate on the watershed by multiplying by wshed1.

(SWE > 0)*SWE to get only positive values

See layout following

- The mean of the SWE layer depicted above is 6.53 inches from the Source tab in layer properties.  This is equivalent to 6.53 x 0.0254 = 0.166 m. The area is 558.4 x 106 m2.   The volume of water present in the form of snow is therefore 0.166 x 558.4 x 106 = 92.62 x 106 m3.